Bio-Oil Geological Storage Pathway — High-Level Overview

Governing framework: Isometric Bio-oil Geological Storage Protocol v1.1. Core accounting equation:

CO₂e_Removal = CO₂e_Stored − CO₂e_Counterfactual − CO₂e_Emissions

Every step below feeds one of those three terms, and net removals are then reduced by a site-specific buffer pool before credits issue.

1. Biomass feedstock

Waste biomass is sourced and screened for eligibility. Its avoided fate (decay, combustion, landfill methane) sets the CO₂e_Counterfactual baseline.

2. Feedstock transport to the production facility

Biomass is moved from its point of origin to the pyrolysis facility. Associated transport emissions enter CO₂e_Emissions.

3. Pyrolysis conversion

Fast pyrolysis converts biomass into bio-oil (the carbon-bearing product), syngas (process heat), and biochar (separate co-product pathway). Pyrolysis process and embodied emissions enter CO₂e_Emissions.

4. Energy use at the production facility

Electricity and on-site fuel consumption during pyrolysis are quantified and added to CO₂e_Emissions.

5. Bio-oil transport to the injection site

Bio-oil is moved from the production facility to the injection well by truck and/or rail. All transport-leg emissions enter CO₂e_Emissions; low-carbon fuel use can be offset 1:1 by BCUs.

6. Pre-treatment and on-site handling

Any conditioning at the injection site (pH adjustment, sparging, etc.) is quantified — both the emissions incurred and any associated mass adjustments to the injectate.

7. Geological injection

Bio-oil is injected via a UIC well into the storage formation (permeable reservoir or salt cavern). Site protocol governs containment, pressure management, and density requirements.

8. Sequestration measurement

Carbon content of the injectate × injectate mass × 44/12 yields CO₂e_Stored. Conservative estimation rules apply per the Isometric Standard.

9. Net removal and buffer pool

CO₂e_Removal is computed from the core equation, then reduced by the site-specific buffer (2% very low risk, 5% low risk) to produce issuable tonnes.

10. Verification and issuance

A VVB verifies the period's claim against the protocol and the project's PDD. Credits issue on the Isometric registry over a 10-year crediting period.

11. Post-injection monitoring

Site-specific monitoring obligations confirm continued containment; the buffer pool absorbs any reversal risk.

Accounting flow in one line: Counterfactual sets the baseline → Stored is what gets injected → Emissions across feedstock sourcing and transport, pyrolysis, energy, bio-oil transport, and injection are subtracted → Buffer is withheld → Net is verified and issued.