Basco 6 EW — Bio-oil / Formation Brine Density-Contrast Analysis

QOWV bio-oil batch (AECN7-2026-W1 / QOWV 7-2026-W2) — Summary for Expert Review

Prepared 2026-07-14 · Charm Industrial MRV · Status: DRAFT for technical review

Units & conversions. g/mL = g/cm³ = kg/L are identical; lab values are reported in g/cm³. In this report, kg/m³ = g/cm³ × 1000 (SI, used for the correction math) and lb/gal = g/cm³ × 8.345 (oilfield unit, used to compare against the permit’s ppg-based inputs — 9.0 ppg pore pressure, 10 lb/gal design density). The 9.0 ppg pore pressure is a pressure gradient expressed as equivalent mud weight, not a fluid density (brine density = 1.01 g/cm³).

1. Question and governing requirement

Does the QOWV bio-oil batch satisfy the density-contrast (buoyant-stability) requirement — is it denser than the Basco 6 formation brine at reservoir conditions, so that it sinks and remains immobile after injection?

Isometric "Bio-oil Storage in Permeable Reservoirs" module, verbatim:

"Density in particular must be greater than that of the formation water."

"ρbio-oil > ρformationfluid where ρ is density at reservoir conditions calculated ρ=m/V ... according to reservoir-specific conditions, such as pressure, temperature and salinity."

The comparison is required at reservoir conditions, not surface. Both densities are measured at surface and corrected downhole.

2. Result

| Condition | QOWV bio-oil | Formation brine | Contrast | Verdict |
| --- | --- | --- | --- | --- |
| Surface | 1050 kg/m³ (1.050) | 1010 kg/m³ (1.010) | +40 kg/m³ | denser |
| Reservoir (~114°F, ~1,640 psi) | 1033 kg/m³ (1.033) | 1004 kg/m³ (1.004) | +29 kg/m³ (+0.029) | SINKS |

Conclusion: QOWV remains denser than the brine at reservoir conditions and will sink. Margin ≈ +29 kg/m³ (≈ +0.24 lb/gal) — thin but positive and structurally robust (§7).

3. Minimum acceptable surface density

Because bio-oil expands more than brine downhole, it must exceed the brine density at surface by a margin to still exceed it at reservoir conditions. Setting bio-oil reservoir density equal to brine reservoir density and solving for surface density gives the binding floor:

| Basis | Minimum surface density | Note |
| --- | --- | --- |
| Naive (beat brine at surface) | 1.010 g/mL (8.43 lb/gal) | Insufficient — ignores differential thermal expansion |
| Reservoir-corrected (binding) | 1.021 g/mL (8.52 lb/gal) | Required so bio-oil ≥ brine at reservoir conditions |

QOWV surface density 1.050 g/mL (8.76 lb/gal) clears the binding floor of 1.021 g/mL by ≈ 0.029 g/mL. The floor scales with assumed reservoir temperature and bio-oil thermal expansion.

4. Sources (measured / documented inputs — click to open)

| Parameter | Value | Source | Status | Link |
| --- | --- | --- | --- | --- |
| Formation brine density | 1.01 g/mL (Sp. Gr., ASTM D-1298-85) | Waypoint Report 24-200-0020, Lab 98021, sampled 7/17/2024 | Measured (verified vs. rendered PDF) | Open |
| Brine composition | TDS 23,300; Cl 24,200; Na 12,600 mg/L | same report | Measured | Open |
| QOWV bio-oil density | 1.050 g/cm³ | Garrett Lutz, Slack 2026-07-13; COA AECN7-2026-W1 / QOWV 7-2026-W2 | Stated on COA (not independently re-read) | Open |
| Formation pore pressure | 9.0 ppg | LDENR Order No. IMD 2025-05 EW, p.4 | Documented (regulatory) | Open |
| Injection interval (perfs) | 3,492–3,534 ft KB (mid ≈ 3,513) | UIC-25 form (in packet); GEC schematic | Measured | Open |
| Base of USDW | 1,170 ft | UIC-25; Order Finding 7 | Documented | Open |
| Reservoir temperature (raw) | Max. Rec. Temp 125°F @ 4,438 ft | Schlumberger openhole log, offset producer Basco 6 #1 (Serial 220147), Run 1 dated 5-16-97 | Measured (log header) | Open |
| MASIP | 455 psig | Order p.4; UIC-25 | Documented | Open |
| Well schematic / perfs (fig.) | Perfs, casing, packer | CHARM INDUSTRIAL FIGURES.pdf (GEC, Fig. 5A–7) | Documented | Open |

5. Assumptions

| Input | Value used | Basis / flag |
| --- | --- | --- |
| Surface temperature (gradient anchor) | ≈70°F | ASSUMPTION — Louisiana mean-annual air temperature. NOT from a project document; used only to convert the measured 125°F @ 4,438 ft into a gradient. |
| Reservoir temperature at 3,513 ft | ≈114°F (46°C) | DERIVED — measured 125°F @ 4,438 ft (offset log, see §4) with the ~70°F surface → 1.25°F/100 ft, extrapolated up to 3,513 ft. Max-recorded temp reads slightly low vs. true static (conservative). |
| Reservoir pressure | ≈1,640 psi (11.3 MPa) | 9.0 ppg pore pressure × 3,513 ft (0.468 psi/ft), documented basis. Consistent with PDD "normally pressured or below." |
| Brine surface ref. temp | 60°F (15.6°C) | ASTM D-1298 convention |
| Bio-oil COA ref. temp | 15°C | ASSUMPTION — COA reference temperature not confirmed |
| Brine thermal expansion β_b | 3.5×10⁻⁴ /°C | Literature, saline water |
| Brine compressibility c_b | 4.4×10⁻¹⁰ /Pa | Literature |
| Bio-oil thermal expansion β_o | 7×10⁻⁴ /°C | Literature, fast-pyrolysis bio-oil — DOMINANT uncertainty |
| Bio-oil compressibility c_o | 5×10⁻¹⁰ /Pa | Literature |
| Correction model | ρ_res = ρ_surf(1 − βΔT + cΔP) | First-order; adequate for small ΔT (≈30°C), ΔP (≈11 MPa) |

6. Calculation

Brine:   1010 × (1 − 3.5e-4×30.4 + 4.4e-10×1.13e+07) = 1010 × 0.99433 = 1004.3 kg/m³

Bio-oil: 1050 × (1 − 7.0e-4×31 + 5.0e-10×1.13e+07) = 1050 × 0.98395 = 1033.1 kg/m³

Reservoir contrast = 1033.1 − 1004.3 = +28.9 kg/m³ (+0.029 g/mL). Positive → sinks.

Minimum surface density = brine reservoir density / bio-oil correction factor = 1004.3 / 0.98395 = 1020.7 kg/m³ = 1.021 g/mL.

7. Sensitivity and robustness

Crossover: holding pressure and coefficients, contrast reaches zero only at a reservoir temperature of ≈248°F (≈120°C) — far above the ≈114°F injection zone. "Sinks" is robust despite the small absolute contrast.

Pressure is a minor lever: it raises both densities nearly equally, so a better pressure input barely moved the contrast.

Dominant uncertainty is the bio-oil thermal-expansion coefficient; a higher value narrows the margin and raises the §3 floor.

8. Residual gaps / recommendations

Reservoir temperature is anchored to an offset-well log plus a ~70°F surface assumption, not a static bottomhole survey on Basco 6. The permit lists "Static reservoir pressure and temperature" as a baseline item; a Basco 6 static temperature would firm the ≈114°F figure and remove the surface assumption.

The bio-oil thermal-expansion coefficient is a literature value. A measured QOWV density at reservoir temperature (or an EOS run) would close the last assumption.

QOWV density (1.050) is stated from the COA via Slack; confirm against the COA of record before use in a verifier submission.

Context: permit application design basis characterized bio-oil at ≈10 lb/gal (≈1.198 g/mL); QOWV at 1.050 (8.76 lb/gal) is a notably lighter batch, which is why it sits near the floor.