Charm — Fort Lupton Pyrolysis Facility

Charm Industrial Bio-Oil Geologic Storage + Range & Plains Biochar Design Document — Production Facilities

Charm — Fort Lupton Pyrolysis Facility (Fort Lupton, CO)

# 1. Site Location

Project Location(s)

* Please submit at least one Address and/or specific geo-coordinates for the project.

* You may submit multiple Project locations – please specify what role each location plays in the Project.

Locations of Pyrolysis:

Charm Industrial, 2360 S Rollie Ave, Fort Lupton, CO 80621

40.076570, -104.804680

Location of Bio Oil Pre-Processing: 838 SW Purity Springs Rd El Dorado, KS 67042 37.813906, -96.914982

Lot 3, Thomas Subdivision, Fort Lupton, CO 80621 (ancillary biomass storage site for Charm pyrolysis) 40.043436, -104.800078

# 2. Environmental and Social Impacts

Please provide an assessment of the environmental and social impacts of the Project, in accordance with Section 3.7 "Environmental and Social Impacts" of the Isometric Standard.

For each aspect of the assessment, demonstrate how the risks have been assessed and if applicable, what mitigation plan is in place to prevent them. If some aspects are not applicable to your project, justify how you determined it. A full Environmental and/or Social Impact Assessment (EIA and/or SIA) is only required if impacts are considered significant and/or if required by the host jurisdiction. | Please provide an assessment of the environmental and social impacts of the Project, in accordance with Section 3.7 “Environmental and Social Impacts” of the Isometric Standard.

| [Optional] I have attached a full EIA document.

| [Optional] I have attached a full SIA document.

X | I acknowledge responsibility for reporting potential environmental and social impacts identified to Isometric and environmental regulators

Environmental Impacts | Environmental Impacts

Resource efficiency and pollution prevention, including pollutant emissions to air, pollutant discharges to water, noise and vibration, generation of waste and release of hazardous materials, chemical pesticides and fertilizers. | Resource efficiency and pollution prevention, including pollutant emissions to air, pollutant discharges to water, noise and vibration, generation of waste and release of hazardous materials, chemical pesticides and fertilizers.

X | Above risks are applicable to this Project

| Not applicable to this Project

Biomass Feedstock and Pyrolysis Charm Bio-oil Charm has gone to significant lengths to ensure that both its biomass feedstock sourcing and pyrolysis processes reflect sustainable best practices and are in compliance with all applicable regulations. Before deciding to move forward with a supplier, Charm assesses the counterfactuals for any given biomass source to confirm there is no alternate near-term use case for the material. The forest residues that are used in this Project come from a state-managed and/or state-funded Fuels Reduction and Forest Restoration project. The project area showed signs of high risk factors for a catastrophic wildfire, thus warranting the Colorado State Forest Service a treatment plan that thins the forest to help increase its health and resilience. Catastrophic wildfires contribute to a mixture of hazardous air pollutants and release large quantities of carbon dioxide and other greenhouse gasses into the atmosphere. By using the woody biomass (fuels) from these fuels reduction projects, Charm can support and reduce the risk by disposing or reducing the fuels in the forest floor while sequestering the biogenic carbon in this material. Charm’s pyrolysis process is designed to recapture waste heat to dry biomass which minimizes the need for combustion of propane or other fuels. Charm has engaged a qualified emissions testing company to perform stack emissions testing and undergone a rigorous third party analysis of potential emissions from its biomass processing and pyrolysis emissions process. This review has confirmed that Charm’s facility emissions are below major air quality permit thresholds.

All Kansas project sites are owned by entities other than Charm with whom Charm has contracted for specific goods or services. Charm’s engagement of these entities for the purpose of this project does not impact land use or acquisition in any material way. That being said, no known involuntary resettlement has occurred due to any third-party commercial activities. Charm leases and operates the Miniforge pyrolysis facility and nearby biomass storage site in Fort Lupton, Co and the Minifinery site in El Dorado, KS where bio oil is pre-processed to ensure suitability for injection. The footprints and usage profiles of these sites are not materially changed due to hosting Charm operations.  No known involuntary resettlement has occurred due to the development of these sites. The Basco 6 Well is leased through a private landowner whose family has owned the property for at least three generations.

# 3. Stakeholder Input Process

Please provide a description and documentation of how comments by local stakeholders have been invited and compiled, a summary of comments received, and report on how due account was taken of comments received. Refer to Section 3.5 “Stakeholder Input Process” of the Isometric Standard for full requirements.

CO Bio-Oil Production Charm Industrial has invested extensive time and resources into initial outreach to the community of Fort Lupton in order to best understand local priorities, challenges, and perceptions of carbon removal and Charm specifically. Charm became an official sponsor of the City of Fort Lupton in January 2024 and joined the Chamber of Commerce in June 2024. These sponsorships have allowed us to introduce ourselves to local businesses, nonprofits, academic institutions, and engaged community members, in addition to actively working with the Fort Lupton Planning Commission on permitting requirements. Local permitting requirements include public comment periods and formal public announcements to give communities a chance to provide input or ask questions. From these events and others, Charm keeps record of common questions and concerns, both from the Fort Lupton community as well as questions gathered at external carbon removal community outreach meetings. Examples of Colorado-based meetings include community listening sessions and outreach events hosted by the National Renewable Energy Lab (NREL), the Colorado Energy Office, and more. The most common questions are based around health and safety, such as clarification on the potential impacts on drinking water, any impacts on air quality, confirmation that bio oil is not combustible, and likelihood of and protocol for spills. Once health and safety concerns are addressed, many follow-up questions are about workforce development opportunities, training opportunities, and local economic impact. Due account has been taken of comments received, and has influenced not only the way the company publicly and locally addresses issues of health and safety, but also the type of local partnerships that Charm is dedicated to building in the community for the long-term. The local priorities and challenges that have been conveyed through these events and relationships will continue to guide Charm toward authentic community partnerships and ultimately the creation of an impactful Community Benefit Agreement.

# 4. Grievance Mechanism

Please outline the mechanism for stakeholders to voice, process and resolve grievances.

Charm Pyrolysis For the Fort Lupton, CO facility, Charm’s basic contact information is available on the Google listing as well as the Fort Lupton Chamber of Commerce website. This phone number goes to our Community Engagement Manager, who can relay the message to the appropriate Charm Industrial employee. Charm has made itself readily available to the local community as a member of the local Chamber of Commerce and as a sponsor of municipal events. Public comment periods are built into all Fort Lupton City Commission permitting processes, which give the public an opportunity to ask questions and voice concerns. These meetings are open to the public and can be attended in person or virtually.