# Measurement, Monitoring, Reporting and Verification (MMRV) Plan — Charm Industrial response to Relae diligence, October 2026

## MMRV plan and activities, including status, current gaps, and estimated date of finalization

The plan is currently active and has been the basis of a third-party validated MRV program whose outputs are also third-party verified monthly.

The plan is a living document: As Charm's operations expand and change, the documentation is updated to reflect current practices on a roughly annual basis with between-update changes noted in GHG Statement Reports issued as part of monthly verification and credit issuance on the Isometric registry.

The most recently updated documentation is the linked sheet, which outlines all parameters that are monitored and measured for the project. This details the nature of the activities and parameters being monitored and measured, the data collected, the calculation methods employed (where applicable) and any adjustments for uncertainty (where applicable).

Tier 2: Combined Monitoring and Emissions Accounting Plan

## Process to select and contract with qualified MMRV partner organizations, including partner or vendor names and anticipated timeline

Charm carefully reviews and vets all MMRV partner organizations. Isometric was chosen for their integrity, including but to limited to rigor of their science based methodologies and support for a CCP-labeled bio-oil geological storage pathway.

## Project metering philosophy and/or detailed metering design report including measurement points, frequency, testing techniques, calibration methods, including post-application monitoring for reversals

Charm's metering follows four principles:

Measure, don't model: mass, carbon content and energy use are measured on every batch or period with the instrument that produced the transaction record (legal-for-trade scales, utility meters, accredited lab analysis).

Conservative by construction: where a measurement carries uncertainty, the removal is reduced, never the emission.

Uncertainty is propagated, not assumed away: each instrument's maximum allowable error is entered in Certify and carried through to the net removal.

Effort follows sensitivity: any parameter that moves net removal by more than 1% when perturbed 20% gets an explicit uncertainty treatment; the rest are used as measured.

[Max to complete: calibration methods]

Measurement points, frequency, testing techniques and uncertainty treatment for every monitored parameter are set out in the linked sheet, one row per parameter. Post-injection monitoring for reversals at the Basco 6 well is in the Storage Monitoring tab: continuous injection, annulus and bottomhole pressure; annual mechanical integrity, pressure fall-off and tracer surveys; semiannual wellhead gas composition against baseline. Any wellhead gas alarm is reported by the operator to Charm within 2 hours, and any identified reversal to Isometric and the verifier within 1–3 business days.

Tier 2: Combined Monitoring and Emissions Accounting Plan

## Credit certification methodology and registries or registration platform(s) including reasoning for selection

Isometric issues Charm's credits under the Bio-oil Geological Storage protocol. The project is "Charm Industrial Great Plains Bio-Oil Sequestration," registered exclusively on the Isometric Registry.

Each credit is a net tonne. Isometric deducts every life-cycle emission and a buffer pool contribution before it issues a credit. Salesforce receives the full net tonnage it pays for, whatever the net-to-gross ratio of a given batch, with the buffer protecting against reversal.

Why Charm chose Isometric:

Buyers pay for verification, so suppliers do not pay their auditors.

Protocols are science-led and go through public comment.

Verification is fast, turning credits around in two weeks or less consistently through 2026.

Every verification report is public on the registry.

A buyer dashboard shows Salesforce its validation status, verification status, and forecast issuance. [Max to confirm]