Bio-oil CI Sensitivity to Emission Sources

Aggregated across 19 closed production periods: PP1–PP8 and August 2025–June 2026

Source: Production Period Close Summaries (Drive folder “PP Close Summaries”). Prepared 2026-09-25.

# Summary

Propane dominates. It is 61.0% of pooled emissions; an 8% change in propane moves bio-oil CI by 5%. No other source moves CI 5% with a change under 30%.

Embodied emissions are second tier. Capital goods (15.8%) needs a 32% change and spare parts & consumables (8.6%) a 58% change to move CI by 5%.

The remaining eight sources cannot plausibly move CI by 5% on their own. After offsets, electricity and biomass transport would need to change by 84% and 110%. Gasoline, waste disposal, tailgas, consumables delivery and water would need changes of 243% to 3,769%. Diesel enters the CI at zero.

Offset coverage matters most for diesel and electricity. BCUs cover 100% of diesel emissions (287.4 of 287.4 t) and RECs cover 93% of electricity emissions (396.9 of 427.1 t). Losing coverage on 8.9% of diesel emissions or 6.4% of electricity emissions would move CI by 5% (Table 2).

# 1. Ranking, share of emissions, and change needed for a 5% CI move

| Rank | Emission source | Pooled emissions in CI (t CO2e) | % of total | Range of % across the 19 periods | Change in source needed to move CI 5% | CI change if source rises 20% (PDD test) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Propane | 311.4 | 61.0% | 33.7–70.5% | 8% | 12.2% |
| 2 | Capital goods (embodied) | 80.7 | 15.8% | 7.3–20.0% | 32% | 3.2% |
| 3 | Spare parts & consumables (embodied) | 43.8 | 8.6% | 4.9–21.6% | 58% | 1.7% |
| 4 | Electricity | 30.2 | 5.9% | 4.3–11.1% | 84% | 1.2% |
| 5 | Biomass transport | 23.1 | 4.5% | 0.0–13.4% | 110% | 0.9% |
| 6 | Gasoline | 10.5 | 2.1% | 0.0–6.2% | 243% | 0.4% |
| 7 | Waste disposal | 5.8 | 1.1% | 0.0–5.5% | 442% | 0.2% |
| 8 | Tailgas | 3.3 | 0.6% | 0.2–3.8% | 777% | 0.1% |
| 9 | Consumables delivery | 0.8 | 0.2% | 0.1–0.3% | 3,091% | 0.0% |
| 10 | Water | 0.7 | 0.1% | 0.0–0.4% | 3,769% | 0.0% |
| 11 | Diesel | 0.0 | 0.0% | 0.0–0.0% | Not applicable (0 in CI) | 0.0% |
|  | Total | 510.3 | 100.0% |  |  |  |

How to read the “change needed” column

5% of pooled emissions is 25.5 t CO2e. Change needed = 5% ÷ source’s share of total. Example: propane 61.03% → 5% ÷ 61.03% = 8.2%. A value above 100% means the source would have to more than double. The same percentage applies whether the change comes from the activity quantity (litres, kWh, tonne-miles) or from the emission factor.

Why one column serves WODO and Aqueous alike

The Jan–Jun 2026 Blended CI Summary describes its method in cell A1 as “pool-then-allocate (carbon-mass weighted)”. Under that method each product’s share of emissions is fixed by carbon mass, not by the emissions themselves. So a change in any source moves the WODO, Aqueous and biochar CI by the same percentage. Pooled CIs across the 19 periods, for reference. Per the PDD v1.12 change note dated 9/9/25, Aqueous received production emissions only from Production Period 7 (June 2025) on, so the pooled Aqueous figure is illustrative, not a CI Charm applies.

| Product | Pooled mass (kg) | Pooled CI (kg CO2e/kg) | 5% of CI (kg CO2e/kg) |
| --- | --- | --- | --- |
| WODO (incl. PP1 dry + wet oil) | 311,758 | 0.7340 | 0.0367 |
| Aqueous | 595,380 | 0.1248 | 0.0062 |

# 2. Sources that enter the CI net of BCUs or RECs

Five lines enter the CI after BCUs or RECs have been subtracted. Their size in Table 1 reflects offset coverage, not the underlying activity. The gross figures below are each workbook’s pre-offset total for that line. “Not possible” means the whole offset is smaller than 5% of pooled emissions, so even losing all of it would not move CI by 5%.

| Source | Offset type | Gross (t CO2e) | Offset (t CO2e) | In CI (t CO2e) | CI rise if all offset lost | Share of offset that must lapse to move CI 5% |
| --- | --- | --- | --- | --- | --- | --- |
| Electricity | RECs | 427.1 | 396.9 | 30.2 | +77.8% | 6.4% |
| Diesel | BCUs | 287.4 | 287.4 | 0.0 | +56.3% | 8.9% |
| Biomass transport | BCUs | 43.7 | 20.6 | 23.1 | +4.0% | Not possible |
| Waste disposal | BCUs | 8.4 | 2.6 | 5.8 | +0.5% | Not possible |
| Consumables delivery | BCUs | 1.7 | 0.9 | 0.8 | +0.2% | Not possible |

Electricity: the 84% in Table 1 assumes RECs keep covering every kWh. A kWh without REC coverage counts at the grid factor instead, about 13 times the solar REC factor (June 2026: 0.4890 vs 0.0366 kg CO2e/kWh, “Emission Factors” E10 and E11). Coverage gaps are the realistic route to a material electricity effect.

# 3. Data notes

Figures used. Each period’s “Consolidated Emissions Inventory (No GHG Neutralization)” block, which is the total each workbook allocates to its products. In all 19 periods the 11 lines sum to the workbook total, and the allocated amounts sum to the same total. For PP1 the “PP1 Summary -- Final Submission” tab is used.

Excluded. The Jul, Aug and Sep 2026 files are unfilled copies of June. Each still has the tab “Jun 26 PP Summary” and the identical total of 27.392 t. Also excluded: the Annual 2025 roll-up, which would double-count, and the Jan–Jun 2026 blend, which is derived from the monthly files.

Biomass transport, PP8 to June 2026. In these 12 workbooks the inventory line takes the pre-BCU total (cell C9, “Total Emissions”). Row 10 (“BCU's Applied”) is still counted in the retired-BCU total in F11 (=Sum(C10,C39,C81,C89)). PP1–PP7 took the post-BCU figure (“Remaining Emissions to Allocate”). Those 12 periods carry 18.4 t of biomass transport that is also covered by BCUs. That is 3.6% of pooled emissions, so the pooled CI is 3.6% higher than the post-BCU treatment would give, and biomass transport ranks 5th instead of 7th.

Cross-check. Repeating the calculation per period and pooling the bio-oil results gives the same ranking. Required changes for the top five sources shift by at most 9 percentage points (biomass transport: 110% pooled vs 101% per period).

# 4. Relation to the sensitivity analysis in PDD v1.12

PDD v1.12 (uncertainty-approach justification) describes the existing test: “the value of each source was arbitrarily increased by 20%… Any parameter that, when increased by 20%, contributed to a change in the estimated removal that was greater than 1% of the original estimation was then subject to an Uncertainty Analysis”. That test was run on “a Sample Calculation of one truckload of bio oil originated at AECN… as well as the first Charm bio-oil injection in Kansas.”

What this document adds. The same kind of test run on 19 periods of Charm production data rather than one sample, measured against production CI rather than net removal, and inverted to give the change needed for a fixed 5% CI move.

Reading the +20% column. It shows each source’s effect on production CI under the PDD’s +20% change. The PDD’s 1% threshold applies to net removal, which also includes transport, injection and storage terms, so the two percentages are not directly comparable.

Aqueous allocation. The PDD change note states: “starting with Production Period 7, which covers June production, we began to allocate emissions to all 3 products.” The workbooks match: Aqueous first appears as an output with allocated emissions in the PP7 summary.

# Appendix: share of each period’s emissions by source

| Period | Total (t CO2e) | Propane | Capital goods | Spare parts & cons. | Electricity | Biomass transport | Gasoline | Waste disposal | Tailgas | Cons. delivery | Water | Diesel |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PP1 | 23.76 | 33.7% | 18.6% | 21.6% | 11.1% | 0.4% | 5.2% | 5.1% | 3.8% | 0.2% | 0.4% | 0.0% |
| PP2 | 29.21 | 59.1% | 13.1% | 13.7% | 9.8% | 0.4% | 2.1% | 0.8% | 0.6% | 0.1% | 0.2% | 0.0% |
| PP3 | 17.16 | 55.4% | 15.5% | 13.0% | 8.0% | 0.4% | 6.2% | 0.4% | 0.7% | 0.2% | 0.3% | 0.0% |
| PP4 | 29.92 | 56.0% | 20.0% | 13.1% | 5.6% | 0.3% | 3.8% | 0.4% | 0.6% | 0.1% | 0.2% | 0.0% |
| PP5 | 35.26 | 59.1% | 19.0% | 11.6% | 5.9% | 0.2% | 3.1% | 0.0% | 0.6% | 0.1% | 0.4% | 0.0% |
| PP6 | 33.92 | 65.6% | 19.2% | 8.0% | 4.9% | 0.1% | 1.5% | 0.0% | 0.6% | 0.1% | 0.1% | 0.0% |
| PP7 | 36.15 | 63.4% | 19.5% | 7.7% | 4.4% | 2.2% | 1.7% | 0.2% | 0.6% | 0.1% | 0.1% | 0.0% |
| PP8 | 43.34 | 66.0% | 15.4% | 6.7% | 4.3% | 6.6% | 0.2% | 0.0% | 0.5% | 0.1% | 0.1% | 0.0% |
| Aug 2025 | 31.56 | 64.0% | 16.9% | 7.1% | 5.3% | 5.2% | 0.7% | 0.0% | 0.5% | 0.1% | 0.1% | 0.0% |
| Sep 2025 | 21.79 | 52.3% | 18.7% | 6.2% | 6.0% | 13.4% | 0.0% | 2.7% | 0.6% | 0.1% | 0.1% | 0.0% |
| Oct 2025 | 24.86 | 64.2% | 12.3% | 6.4% | 5.7% | 5.8% | 2.0% | 3.0% | 0.4% | 0.1% | 0.0% | 0.0% |
| Nov 2025 | 22.09 | 64.5% | 7.3% | 5.5% | 6.1% | 8.3% | 2.3% | 5.5% | 0.2% | 0.2% | 0.1% | 0.0% |
| Dec 2025 | 16.65 | 60.7% | 11.3% | 4.9% | 5.5% | 13.0% | 2.2% | 1.8% | 0.3% | 0.2% | 0.0% | 0.0% |
| Jan 2026 | 21.54 | 61.1% | 13.6% | 5.3% | 6.0% | 11.6% | 1.9% | 0.0% | 0.4% | 0.1% | 0.0% | 0.0% |
| Feb 2026 | 22.74 | 62.9% | 13.8% | 5.6% | 5.8% | 7.9% | 2.3% | 1.1% | 0.4% | 0.1% | 0.1% | 0.0% |
| Mar 2026 | 26.03 | 59.8% | 16.5% | 6.6% | 5.4% | 8.6% | 1.7% | 0.5% | 0.5% | 0.3% | 0.0% | 0.0% |
| Apr 2026 | 25.73 | 66.0% | 16.1% | 6.4% | 5.0% | 3.8% | 1.9% | 0.0% | 0.5% | 0.3% | 0.1% | 0.0% |
| May 2026 | 21.16 | 66.1% | 16.0% | 6.1% | 5.7% | 0.0% | 1.2% | 4.1% | 0.5% | 0.3% | 0.1% | 0.0% |
| Jun 2026 | 27.39 | 70.5% | 10.9% | 6.2% | 4.7% | 5.4% | 1.7% | 0.0% | 0.3% | 0.3% | 0.1% | 0.0% |
| Pooled | 510.26 | 61.0% | 15.8% | 8.6% | 5.9% | 4.5% | 2.1% | 1.1% | 0.6% | 0.2% | 0.1% | 0.0% |